BK (ABK) TRANSPONDER


Description.-The ABK transpondor (known as BK in ship installations) is a combination receiver and transmitter which accepts challenging signals from an interrogator and automatically answers this challenge in coded form for identification purposes. It is used to reply to challenges from Mark III IFF interrogators. 
Power supply.-ABK-1-21 to 30 volts d.c. 5 amp. 
Transmitter specifications:
Frequency (Mc) (sweep)  158 to 186.
R-f source              7193 tube.

R-f lines               RG-12/U.

Pulser type             hard tube.

R-f peak power          4 watts.

Pulse rates             up to 2000.

Pulse lengths (micro second) 7, 21, 80.
Receiver specifications:

Type                    Superregenerative.

Sensitivity             84 to 70 db.
Antenna specifications.-"Steering wheel" or "life preserver" type. 
Tube complement.-2 type 6H6, 6 type 6SH7, 2 type 7193 or 1 type 7193 and 1 type 2622. 

         Dimensions and weights

   Unit   Navy Type No. Height Width  Depth      Weight

                        Inches Inches Inches     Pounds

ABK            43AAX   12 5/16   9     12 31/32    30

ABK-1          43AAY

Control unit   23ABG    3 1/2    8      6 1/4       1.0

Selector unit  23ABH    3 1/2    8      6 1/4        .5
Literature.-Instruction Book-CSP 1375. 

CW-23-ABH Selector Unit, 3 1/2" x 8" x 6 1/4", approx. .5 lbs

[image: image1.png]



[image: image2.png]


[image: image3.png]



IFF Destructor Switch Type N-17-S (or M-17-S)

[image: image4.jpg]



[image: image5.jpg]AN POSITION

2355107127
itorror Sw INDIATOR LIGHT

W35/
£6-100
ir-5039

el
e W-305

7o w105 ponice
2ev.oc.

BN T,
pd





[image: image6.jpg]



REFERENCES:

The information enclosed here is excerpted from: 
Catalogue of Naval Electronic Equipment-April 1946- NavShips 900,116. 

ABK

Transponder, Identification friend or foe, Airborne, Mk III, 157-187 MHz, 6 W, Similar to SCR-595(*) and AN/APX-5(*), made by Zenith, CZR-43AAY (24 V) and Hazelton, CHZ-43AAX (12 V), 900909, 1943, Uses British-style power and antenna connectors, 14/28 VDC @ 120 W, 33 Lbs. 



ABK-1

Transponder, Identification friend or foe, Airborne, Similar to ABK except 24 VDC, Hazeltine, 1942. 



ABK-2

Transponder, Identification friend or foe, Airborne, Similar to ABK except component parts, 14 VDC, Stromberg-Carlson, 1942. 



ABK-3

Transponder, Identification friend or foe, Airborne, Similar to ABK-2 except 24 VDC, Stromberg-Carlson, 1942. 



ABK-4

Transponder, Identification friend or foe, Airborne, Similar to ABK, 14 VDC, Zenith, 1942. 



ABK-5

Transponder, Identification friend or foe, Airborne, Similar to ABK except 24 VDC, Zenith, 1942. 



ABK-6

Transponder, Identification friend or foe, Airborne, Similar to ABK, 14 VDC, Stewart-Warner, 1942. 



ABK-7

Transponder, Identification friend or foe, Airborne, Similar to ABK except 24 VDC, Stewart-Warner, 1942.
Below is a list of plugs and brackets needed to complete the ABK-1 IFF system "D" Plug Holder, NAF 214848-1, MC-277, SA-5085 (1 needed) 
2 Female "D" Plug, NAF 68969-2, PL-177, SO-5007 & PP-5016 (1 needed) 
5 Pin Female Plug, NAF 47908-1, PL-265, SO-5004 & PP-5015 (1 needed) 
7 Pin Female Plug, NAF 68925-1, PL-183, SO-5005 & PP-5014 (2 are needed) 
Ant Plug, NAF 47848-1, MC-277, SA-5079 (1 needed) 
Ant Plug, NAF 47848-2, MC-320, SA-5080 (1 needed) 
Cable Grip, NAF 3111145-1, SA-5081 (2 needed) 
Battery Cable Assembly SA-5063 (1) 
Control Unit Mounting FT-248-A, SA-5015 (1 needed) 
Fuse Assembly SA-5086 (1 needed) 
Junction Box Assembly NAF68969-1, SO-5006 (1 needed) 

[image: image7.png]T “m

) - Qm

2 %ymw et
2

mI:@@_
RALEI=r




[image: image8.png]



[image: image9.jpg]



[image: image10.jpg]



