}`HPLJIIP}.cw10

.rm6.5"

.ul on

.op

TO:       Bill Henry

Ken Sartain

Rex Debolt

FROM:     A.B. White

SUBJECT:  ST-8000 V2.0 - FAST/SLOW TUNING RATE

DATE:     March 20, 1991

.fo ABW  3-20-91  stm_v20.doc

In response to our customer's requirement for fast and slow tuning rate

options, a new switch is added to the ST-8000 front panel to select a 1 Hz or

10 Hz tuning rate.

ST-8000 V2.0 firmware sets the tuning rate to FAST (10 Hz) when the switch is

UP and SLOW (1 Hz) when the switch is DOWN.

The current state of the switch is shown on the remote port STATUS page after

the PRINT option setting.

This firware will only work properly in an ST-8000 with a "new" front panel

display board.  The original boards were designed for a push button switch,

while the new display boards are designed for a SPDT toggle switch next to the

main tuning knob.  If the switch is not installed, the tuning rate is always

set for the SLOW (1 Hz) rate.

ST-8000  V2.0  Checksum:  U10  1400

U11  B900